API for 3rd party

v.1.0

API

Assign IP: Stages leave the factory with the IP 192.168.64.100

Port: 7621

| STRING TO SEND | DESCRIPTION | VARIABLE VALUES |
| --- | --- | --- |
| POWER ON | NORMAL MODE | NONE |
| POWER OFF | STANDBY MODE | NONE |
| SET ZONE-X.MUTE Y\r\n | IF Y=1 MUTES ZONE X. IF Y=0 UNMUTES ZONE X | X[A,B,C,D] , Y[0,1] |
| SET ZONE-X.GAIN Y\r\n | SET ZONE X TO VOLUME=Y. Y CAN BE FROM MIN TO MAX | X[A,B,C,D] , Y[INT(MIN) to INT(MAX)] |
| SET OUT-X.GAIN Y\r\n | SET OUTPUT X TO VOLUME=Y | X[1,2,3,4] , Y[-30dB to 15dB] |
| SET OUT-X.MUTE Y\r\n | IF Y=1 MUTES OUTPUT X. IF Y=0 UNMUTES OUTPUT X | X[1,2,3,4] , Y[0,1] |
| SET OUT-X. SRC Y" | LINK OUT X TO ZONE Y | X[1,2,3,4] , Y[A,B,C,D] |

EXAMPLE WITH HERCULES UTILITY

IN THIS PICTURE WE SEE:

SELECTED TCP CLIENT

DESTINATION IP (POWER AMPLIFIER): 192.168.64.100

DESTINATION PORT:7621

ALL STRINGS ARE FINISHED WITH $0d$0A THAT IS THE HEXADECIMAL REPRESENTATION OF \r\n.

FIRST COMMAND: SET ZONE D TO MUTE

SECOND COMMAND: SET ZONE D TO UNMUTE

THIRD COMMAND: SET ZONE D GAIN TO –10 dB