ARENA SERIES                       (IP54)

v.1.0

ARENA 6CX

ARENA 8CX

ARENA 10CX

ARENA 12CX

ARENA 28S

OVERVIEW

The ARENA Series compact coaxial loudspeaker system is designed to provide high quality sound reproduction in applications requiring extreme weather resistance. All cabinets are built from water-resistant polyvinyl board with tough texture coating. The hydrofobically-treated grill, moisture-sealed crossover and waterproof connector makes it reliable under long-term exposure to tough environmental conditions.

The ARENA Series consists of full range 6”/8”/10”/12” coaxial speakers and a dual 8” subwoofer.

Ideal for stadiums fairgrounds, outdoor entertainment centers, athletic fields, swimming pools background music, worships, multipurpose outdoor and indoor venues, etc.

Technical Data:

|  | ARENA 6CX | ARENA 8CX | ARENA 10CX | ARENA 12CX | ARENA 28S |
| --- | --- | --- | --- | --- | --- |
| Power (Passive) | 300W | 360W | 400W | 600W | 800W |
| Impedance | 8Ω Nominal |  |  |  | 4Ω Nominal |
| 70V | 60W/30W/15W/7.5W | 120W/60W/30W/15W | 200W/100W/50W/25W | - | - |
| 100V | 60W/30W/15W | 120W/60W/30W | 200W/100W/50W | - | - |
| Transducer | LF: 1 x 6.5”, HF: 1” Compression driver (Coaxial) | LF: 1 x 8”, HF: 1” Compression driver (Coaxial) | LF: 1 x 10”, HF: 1-3/8” Compression driver (Coaxial) | LF: 1 x 12”, HF: 1-3/4” Compression driver (Coaxial) | LF: 2 x 8” band-pass |
| Connectors | Waterproof 4 pin connection system |  |  |  | 2 x 4 Pin XLN Speakon |
| Frequency response | 80Hz – 20KHz | 80Hz – 20KHz | 70Hz – 20KHz | 65Hz – 20KHz | 45Hz – 200Hz |
| Sensitivity | 97dB | 100dB | 105dB | 107dB | 95dB |
| SPL Max | 125dB | 128dB | 133dB | 135dB | 125dB |
| Dispersion | 110° x 110° (-10dB) | 110° x 110° (-10dB) | 95° x 95° (-10dB) | 60° x 60° (-10dB) | Omnidirectional |
| Cabinet construction | Water-resistant polyvinyl |  |  |  |  |
| Mounting | U Bracket |  |  |  | Metal stand mount on top |
| Finish | Texture coating |  |  |  |  |
| Grill | Steel with hydrofobically treated |  |  |  |  |
| Measures W/H/D | 256mm 275mm 251mm | 299mm 310mm 299mm | 330mm 370mm 330mm | 409mm 450mm 409mm | 208mm 450mm 670mm |
| Weight | 9.2Kg | 11.5Kg | 15.3Kg | 19,5Kg | 22.8Kg |

FEATURES:

- Customized high performance woofer & compression driver.

- Built-in 70V/100V transformer with power tap switch (ARENA 6CX, 8CX & 10CX).

- Made of water resistant polyvinyl. IP54.

- Hidrophobically-treated Grill.

- Waterproof strip connector.

- U-bracket included for installation (ARENA 6CX, 8CX, 10CX & 12CX).

CONNECTIONS

INSTALLATION

CAUTIONS

To connect to a 70/100V line speaker, select the desired power corresponding to its power, to connect to a low impedance speaker, select 8Ω.

MEASURES (mm):

ARENA 6CX

ARENA 8CX

ARENA 10CX

ARENA 12CX

ARENA 28S

DESCRIPCIÓN GENERAL

La Serie ARENA de altavoces coaxiales compactos está diseñada para proporcionar una reproducción de sonido de alta calidad en aplicaciones que requieren una resistencia extrema a la intemperie. Todos los recintos están fabricados con tablero de polivinilo resistente al agua con un revestimiento de textura de alta calidad. La rejilla tratada hidrófobamente, el crossover sellado contra la humedad y el conector impermeable hacen que sea fiable bajo una exposición prolongada a duras condiciones ambientales.

La Serie ARENA consta de altavoces coaxiales de rango completo de 6"/8"/10"/12" y un subwoofer doble de 8".

Ideal para estadios, recintos feriales, centros de ocio al aire libre, campos de atletismo, piscinas, música de fondo, lugares de culto, espacios polivalentes de interior y exterior, etc.

Datos Técnicos:

|  | ARENA 6CX | ARENA 8CX | ARENA 10CX | ARENA 12CX | ARENA 28S |
| --- | --- | --- | --- | --- | --- |
| Potencia (Pasiva) | 300W | 360W | 400W | 600W | 800W |
| Impedancia | 8Ω Nominal |  |  |  | 4Ω Nominal |
| 70V | 60W/30W/15W/7.5W | 120W/60W/30W/15W | 200W/100W/50W/25W | - | - |
| 100V | 60W/30W/15W | 120W/60W/30W | 200W/100W/50W | - | - |
| Transductor | LF: 1 x 6.5”, HF: 1” Compresión driver (Coaxial) | LF: 1 x 8”, HF: 1” Compresión driver (Coaxial) | LF: 1 x 10”, HF: 1- Compresión driver (Coaxial) | LF: 1 x 12”, HF: 1-3/4” Compresión driver (Coaxial) | LF: 2 x 8” Pasa Banda |
| Conexionado | Sistema de conexión impermeable de 4 pines |  |  |  | 2 x 4 Pin XLN Speakon |
| Respuesta en Frecuencia | 80Hz – 20KHz | 80Hz – 20KHz | 70Hz – 20KHz | 65Hz – 20KHz | 45Hz – 200KHz |
| Sensibilidad | 97dB | 100dB | 105dB | 107dB | 95dB |
| SPL Max | 125dB | 128dB | 133dB | 135dB | 125dB |
| Dispersión | 110° x 110° (-10dB) | 110° x 110° (-10dB) | 95° x 95° (-10dB) | 60° x 60° (-10dB) | Omnidireccional |
| Construcción del recinto | Polivinilo resistente al agua |  |  |  |  |
| Montaje | Soporte a pared en U |  |  |  | Soporte metálico superior |
| Acabado | Revestimiento texturizado |  |  |  |  |
| Rejilla | Acero con tratamiento hidrofóbico |  |  |  |  |
| Medidas An/AL/Pr | 256mm 275mm 251mm | 299mm 310mm 299mm | 330mm 370mm 330mm | 409mm 450mm 409mm | 208mm 450mm 670mm |
| Peso | 9.2Kg | 11.5Kg | 15.3Kg | 19,5Kg | 22.8Kg |

Características:

- Woofer y driver de compresión de alto rendimiento.

- Transformador de 70 V/100 V incorporado con interruptor de selección de potencia (ARENA 6CX, 8CX & 10CX).

- Polivinilo resistente al agua. IP54.

- Rejilla con tratamiento hidrofóbico.

- Conector impermeable.

- Soporte en U incluido para la instalación (ARENA 6CX, 8CX, 10CX & 12CX).

CONEXIONES

INSTALACIÓN

PRECAUCIONES

Para conectar a altavoz a línea 70/100V seleccione a la potencia deseada correspondiente a su potencia, para conectar a altavoz en baja impedancia selecciona

MEASURES:

ARENA 6CX

ARENA 8CX

ARENA 10CX

ARENA 12CX

ARENA 28S